EXP-1

CODE:

clc; clear; close all;

Fs = 1000;

Ap = 1;

As = 40;

% LOW-PASS FILTER

[Nlp,Wnlp] = buttord(100/(Fs/2),150/(Fs/2),Ap,As);

[blp,alp] = butter(Nlp,Wnlp,'low');

% HIGH-PASS FILTER

[Nhpf,Wnhp] = buttord(250/(Fs/2),200/(Fs/2),Ap,As);

[bhp,ahp] = butter(Nhpf,Wnhp,'high');

% COMMAND WINDOW OUTPUT

fprintf('\n========================================\n');

fprintf(' BUTTERWORTH IIR DIGITAL FILTER DESIGN\n');

fprintf('========================================\n');

fprintf('\nLOW-PASS FILTER\n');

fprintf('Order = %d\n',Nlp);

fprintf('Cutoff Frequency = %.2f Hz\n',Wnlp*Fs/2);

fprintf('Numerator Coefficients:\n');

disp(blp);

fprintf('Denominator Coefficients:\n');

disp(alp);

fprintf('\nHIGH-PASS FILTER\n');

fprintf('Order = %d\n',Nhpf);

fprintf('Cutoff Frequency = %.2f Hz\n',Wnhp*Fs/2);

fprintf('Numerator Coefficients:\n');

disp(bhp);

fprintf('Denominator Coefficients:\n');

disp(ahp);

% FREQUENCY RESPONSES

[Hlp,f] = freqz(blp,alp,2048,Fs);

[Hhp,~] = freqz(bhp,ahp,2048,Fs);

% GRAPH 1: LOW-PASS FILTER

figure('Color','w','Name','Low-Pass Filter');

subplot(2,1,1);

plot(f,20*log10(max(abs(Hlp),1e-10)),'b','LineWidth',1.8);

grid on; xlim([0 Fs/2]); ylim([-100 5]);

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Butterworth IIR Low-Pass Magnitude Response');

xline(100,'--g','Passband Edge');

xline(150,'--r','Stopband Edge');

subplot(2,1,2);

plot(f,unwrap(angle(Hlp))*180/pi,'r','LineWidth',1.5);

grid on; xlim([0 Fs/2]);

xlabel('Frequency (Hz)');

ylabel('Phase (Degrees)');

title('Low-Pass Phase Response');

% GRAPH 2: HIGH-PASS FILTER

figure('Color','w','Name','High-Pass Filter');

subplot(2,1,1);

plot(f,20*log10(max(abs(Hhp),1e-10)),'b','LineWidth',1.8);

grid on; xlim([0 Fs/2]); ylim([-100 5]);

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Butterworth IIR High-Pass Magnitude Response');

xline(200,'--r','Stopband Edge');

xline(250,'--g','Passband Edge');

subplot(2,1,2);

plot(f,unwrap(angle(Hhp))*180/pi,'r','LineWidth',1.5);

grid on; xlim([0 Fs/2]);

xlabel('Frequency (Hz)');

ylabel('Phase (Degrees)');

title('High-Pass Phase Response');

fprintf('\nFilter design and graph plotting completed.\n');

OUTPUT:

========================================

BUTTERWORTH IIR DIGITAL FILTER DESIGN

========================================

LOW-PASS FILTER

Order = 12

Cutoff Frequency = 106.35 Hz

Numerator Coefficients:

1.0e-03 *

0.0002    0.0027    0.0146    0.0488    0.1098    0.1757    0.2050    0.1757    0.1098    0.0488    0.0146    0.0027    0.0002

Denominator Coefficients:

1.0000   -6.8844   22.4341  -45.5100   63.7725  -64.8501   48.9643  -27.6092   11.5218   -3.4662    0.7128   -0.0899    0.0053

HIGH-PASS FILTER

Order = 17

Cutoff Frequency = 242.27 Hz

Numerator Coefficients:

Columns 1 through 17

0.0001   -0.0013    0.0100   -0.0502    0.1756   -0.4566    0.9132   -1.4350    1.7938   -1.7938    1.4350   -0.9132    0.4566   -0.1756    0.0502   -0.0100    0.0013

Column 18

-0.0001

Denominator Coefficients:

Columns 1 through 17

1.0000   -0.5247    2.4322   -1.0556    2.2461   -0.7954    1.0097   -0.2858    0.2357   -0.0517    0.0283   -0.0046    0.0016   -0.0002    0.0000   -0.0000    0.0000

Column 18

-0.0000

Filter design and graph plotting completed.

>>